|  | University of Southampton · School of Chemistry # EPSRC National Crystallography Service |  |
| --- | --- | --- |

Table 1. Crystal data and structure refinement.

Identification code 	05src0044

Empirical formula 	C5H4N4O3

Formula weight 	168.12

Temperature 	120(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P21/n

Unit cell dimensions	a = 6.2302(19) Å	a = 90°

b = 7.2368(11) Å	b = 90.85(4)°

c = 13.113(12) Å	g  = 90°

Volume	591.2(6) Å3

Z	4

Density (calculated)	1.889 Mg / m3

Absorption coefficient	0.160 mm-1

F(000)	344

Crystal	Slab; Colourless

Crystal size	0.10 ´ 0.07 ´ 0.02 mm3

q range for data collection	3.22 - 27.48°

Index ranges	-8 £ h £ 7, -8 £ k £ 9, -17 £ l £ 14

Reflections collected	5439

Independent reflections	1348 [Rint = 0.0481]

Completeness to q = 27.48°	99.3 %

Absorption correction	Semi-empirical from equivalents

Max. and min. transmission	0.9968 and 0.9842

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	1348 / 0 / 126

Goodness-of-fit on F2	1.174

Final R indices [F2 > 2s(F2)]	R1 = 0.0675, wR2 = 0.1731

R indices (all data)	R1 = 0.0771, wR2 = 0.1780

Extinction coefficient	0.085(13)

Largest diff. peak and hole	0.380 and -0.346 e Å-3

Diffractometer: Nonius KappaCCD area detector (f scans and w scans to fill asymmetric unit sphere). Cell determination: DirAx (Duisenberg, A.J.M.(1992). J. Appl. Cryst. 25, 92-96.) Data collection: Collect (Collect: Data collection software, R. Hooft, Nonius B.V., 1998). Data reduction and cell refinement: Denzo (Z. Otwinowski & W. Minor, Methods in Enzymology (1997) Vol. 276: Macromolecular Crystallography, part A, pp. 307-326; C. W. Carter, Jr. & R. M. Sweet, Eds., Academic Press). Absorption correction: SORTAV (R. H. Blessing, Acta Cryst. A51 (1995) 33-37; R. H. Blessing, J. Appl. Cryst. 30 (1997) 421-426). Structure solution: SHELXS97 (G. M. Sheldrick, Acta Cryst. (1990) A46 467-473). Structure refinement: SHELXL97 (G. M. Sheldrick (1997), University of Göttingen, Germany). Graphics: ORTEP3 for Windows (L. J. Farrugia, J. Appl. Crystallogr. 1997, 30, 565).

Special details:

The hydrogen atoms were placed in idealised positions but were then allowed to be freely refined.

Table 2. Atomic coordinates [´ 104], equivalent isotropic displacement parameters [Å2 ´ 103] and site occupancy factors. Ueq is defined as one third of the trace of the orthogonalized Uij tensor.

Atom 	x	y	z	Ueq	S.o.f.

C1	-3581(5)	1114(4)	3815(2)	20(1)	1

C2	-511(5)	2651(4)	3649(2)	18(1)	1

C3	-720(5)	2172(4)	4650(2)	19(1)	1

C4	858(5)	2661(4)	5383(2)	19(1)	1

C5	2752(5)	4201(4)	3954(3)	20(1)	1

N1	-2267(4)	2004(4)	3132(2)	18(1)	1

N2	-2642(4)	1246(4)	4750(2)	19(1)	1

N3	2553(4)	3637(4)	4941(2)	19(1)	1

N4	1149(4)	3652(4)	3286(2)	20(1)	1

O1	-5318(4)	325(3)	3597(2)	22(1)	1

O2	819(4)	2317(3)	6303(2)	23(1)	1

O3	4264(4)	5159(3)	3676(2)	24(1)	1

Table 3. Bond lengths [Å] and angles [°].

C1-O1	1.253(4)

C1-N2	1.354(4)

C1-N1	1.382(4)

C2-N4	1.355(4)

C2-N1	1.361(4)

C2-C3	1.366(5)

C3-N2	1.381(4)

C3-C4	1.410(4)

C4-O2	1.232(4)

C4-N3	1.403(4)

C5-O3	1.229(4)

C5-N3	1.364(4)

C5-N4	1.377(4)

N1-H1	0.94(4)

N2-H2	0.90(4)

N3-H3	0.95(5)

N4-H4	0.84(5)

O1-C1-N2	126.8(3)

O1-C1-N1	125.7(3)

N2-C1-N1	107.6(3)

N4-C2-N1	128.4(3)

N4-C2-C3	123.9(3)

N1-C2-C3	107.7(3)

C2-C3-N2	108.1(3)

C2-C3-C4	121.1(3)

N2-C3-C4	130.9(3)

O2-C4-N3	122.2(3)

O2-C4-C3	126.3(3)

N3-C4-C3	111.5(3)

O3-C5-N3	122.1(3)

O3-C5-N4	121.7(3)

N3-C5-N4	116.2(3)

C2-N1-C1	108.5(3)

C2-N1-H1	126(2)

C1-N1-H1	125(2)

C1-N2-C3	108.1(3)

C1-N2-H2	130(3)

C3-N2-H2	122(3)

C5-N3-C4	128.5(3)

C5-N3-H3	122(3)

C4-N3-H3	110(3)

C2-N4-C5	118.8(3)

C2-N4-H4	127(3)

C5-N4-H4	113(3)

Symmetry transformations used to generate equivalent atoms:

Table 4. Anisotropic displacement parameters [Å2´ 103]. The anisotropic displacement

factor exponent takes the form: -2p 2[h2a*2U11 + ... + 2 h k a* b* U12 ].

Atom	U11	U22	U33	U23	U13	U12

C1	20(2) 	21(2)	18(2) 	1(1)	0(1) 	1(1)

C2	20(2) 	17(1)	17(2) 	-2(1)	-1(1) 	4(1)

C3	19(2) 	19(1)	20(2) 	-1(1)	-1(1) 	0(1)

C4	20(2) 	17(1)	20(2) 	-1(1)	1(1) 	1(1)

C5	19(2) 	20(2)	23(2) 	-3(1)	-2(1) 	2(1)

N1	18(1) 	21(1)	16(1) 	-1(1)	1(1) 	0(1)

N2	19(1) 	22(1)	18(1) 	1(1)	1(1) 	0(1)

N3	17(1) 	20(1)	21(1) 	-3(1)	-4(1) 	-1(1)

N4	19(1) 	24(1)	18(1) 	1(1)	-2(1) 	-1(1)

O1	21(1) 	27(1)	19(1) 	-1(1)	1(1) 	-3(1)

O2	25(1) 	27(1)	16(1) 	-2(1)	-2(1) 	1(1)

O3	20(1) 	28(1)	23(1) 	-1(1)	-2(1) 	-4(1)

Table 5. Hydrogen coordinates [´ 104] and isotropic displacement parameters [Å2 ´ 103].

Atom 	x	y	z	Ueq	S.o.f.

H1	-2610(60)	2240(50)	2440(30)	20(9)	1

H2	-3070(60)	800(60)	5350(30)	26(10)	1

H3	3590(70)	3950(60)	5450(40)	35(11)	1

H4	1270(70)	4090(60)	2700(40)	32(11)	1

Table 6. Hydrogen bonds [Å and °].

D-H···A	d(D-H)	d(H···A)	d(D···A)	Ð(DHA)

N1-H1...O2i	0.94(4)	1.81(4)	2.708(4)	160(3)

N2-H2...O1ii	0.90(4)	1.90(4)	2.772(4)	163(4)

N3-H3...O3iii	0.95(5)	1.86(5)	2.806(4)	172(4)

N4-H4...O1iv	0.84(5)	2.00(5)	2.792(4)	157(4)

Symmetry transformations used to generate equivalent atoms:

(i) x-1/2,-y+1/2,z-1/2    (ii) -x-1,-y,-z+1    (iii) -x+1,-y+1,-z+1

(iv)  -x-1/2,y+1/2,-z+1/2