|  | # Single Crystal X-Ray Diffraction Service School of Chemistry - University of Southampton Contact: Dr Mark E Light, light@soton.ac.uk, ex 29429 # Single Crystal X-Ray Diffraction Service School of Chemistry - University of Southampton Contact: Dr Mark E Light, light@soton.ac.uk, ex 29429 |
| --- | --- |

Table 1. Crystal data and structure refinement details.

Identification code 	2009src0664

Empirical formula 	C17H14O

Formula weight 	234.28

Temperature 	120(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	C2/c

Unit cell dimensions	a = 13.3991(5) Å	 = 90°

b = 8.2341(3) Å	 = 96.356(2)°

c = 22.8014(8) Å	  = 90°

Volume	2500.20(16) Å3

Z	8

Density (calculated)	1.245 Mg / m3

Absorption coefficient	0.076 mm1

F(000)	992

Crystal	block; colourless

Crystal size	0.30  0.30  0.10 mm3

 range for data collection	2.99  27.47°

Index ranges	17  h  17, 10  k  10, 29  l  29

Reflections collected	17562

Independent reflections	2874 [Rint = 0.0602]

Completeness to  = 27.47°	99.9 %

Absorption correction	Semiempirical from equivalents

Max. and min. transmission	0.9925 and 0.9776

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	2874 / 0 / 165

Goodness-of-fit on F2	1.046

Final R indices [F2 > 2(F2)]	R1 = 0.0553, wR2 = 0.1463

R indices (all data)	R1 = 0.0821, wR2 = 0.1638

Largest diff. peak and hole	0.270 and 0.284 e Å3

Diffractometer: Nonius KappaCCD area detector ( scans and  scans to fill asymmetric unit ). Cell determination: DirAx (Duisenberg, A.J.M.(1992). J. Appl. Cryst. 25, 92-96.) Data collection: Collect (Collect: Data collection software, R. Hooft, Nonius B.V., 1998). Data reduction and cell refinement: Denzo (Z. Otwinowski & W. Minor, Methods in Enzymology (1997) Vol. 276: Macromolecular Crystallography, part A, pp. 307326; C. W. Carter, Jr. & R. M. Sweet, Eds., Academic Press). Absorption correction: Sheldrick, G. M. SADABS - Bruker Nonius area detector scaling and absorption correction - V2.10 Structure solution: SHELXS97 (G. M. Sheldrick, Acta Cryst. (1990) A46 467473). Structure refinement: SHELXL97 (G. M. Sheldrick (1997), University of Göttingen, Germany). Graphics: Cameron - A Molecular Graphics Package. (D. M. Watkin, L. Pearce and C. K. Prout, Chemical Crystallography Laboratory, University of Oxford, 1993).

Special details:

Table 2. Atomic coordinates [ 104], equivalent isotropic displacement parameters [Å2  103] and site occupancy factors. Ueq is defined as one third of the trace of the orthogonalized Uij tensor.

Atom 	x	y	z	Ueq	S.o.f.

C1	6312(1)	3436(2)	6611(1)	25(1)	1

C2	5913(1)	4631(2)	6933(1)	30(1)	1

C3	6520(1)	5838(2)	7221(1)	33(1)	1

C4	7528(1)	5845(2)	7183(1)	32(1)	1

C5	9022(1)	4677(2)	6810(1)	30(1)	1

C6	9469(1)	3510(2)	6507(1)	31(1)	1

C7	8881(1)	2258(2)	6219(1)	28(1)	1

C8	7857(1)	2191(2)	6228(1)	23(1)	1

C9	7371(1)	3397(2)	6547(1)	22(1)	1

C10	7969(1)	4651(2)	6846(1)	26(1)	1

C11	7318(1)	805(2)	5907(1)	24(1)	1

C12	7393(1)	760(2)	6144(1)	28(1)	1

C13	6936(1)	2067(2)	5841(1)	29(1)	1

C14	6391(1)	1873(2)	5288(1)	27(1)	1

C15	6319(1)	315(2)	5048(1)	29(1)	1

C16	6772(1)	998(2)	5351(1)	28(1)	1

C17	5891(1)	3307(2)	4971(1)	36(1)	1

O1	5648(1)	2296(2)	6376(1)	35(1)	1

Table 3. Bond lengths [Å] and angles [°].

C1O1	1.362(2)

C1C2	1.371(2)

C1C9	1.443(2)

C2C3	1.400(3)

C2H2	0.9500

C3C4	1.364(3)

C3H3	0.9500

C4C10	1.415(2)

C4H4	0.9500

C5C6	1.361(3)

C5C10	1.423(2)

C5H5	0.9500

C6C7	1.414(2)

C6H6	0.9500

C7C8	1.376(2)

C7H7	0.9500

C8C9	1.429(2)

C8C11	1.498(2)

C9C10	1.432(2)

C11C12	1.396(2)

C11C16	1.402(2)

C12C13	1.385(2)

C12H12	0.9500

C13C14	1.396(2)

C13H13	0.9500

C14C15	1.393(2)

C14C17	1.503(2)

C15C16	1.386(2)

C15H15	0.9500

C16H16	0.9500

C17H17A	0.9800

C17H17B	0.9800

C17H17C	0.9800

O1H1	0.8400

O1C1C2	115.46(15)

O1C1C9	123.42(14)

C2C1C9	121.10(16)

C1C2C3	121.31(16)

C1C2H2	119.3

C3C2H2	119.3

C4C3C2	120.06(16)

C4C3H3	120.0

C2C3H3	120.0

C3C4C10	120.49(16)

C3C4H4	119.8

C10C4H4	119.8

C6C5C10	120.92(16)

C6C5H5	119.5

C10C5H5	119.5

C5C6C7	119.77(16)

C5C6H6	120.1

C7C6H6	120.1

C8C7C6	121.87(16)

C8C7H7	119.1

C6C7H7	119.1

C7C8C9	119.41(15)

C7C8C11	116.79(14)

C9C8C11	123.79(14)

C8C9C10	118.75(14)

C8C9C1	125.15(15)

C10C9C1	116.07(14)

C4C10C5	119.79(16)

C4C10C9	120.95(15)

C5C10C9	119.26(15)

C12C11C16	117.60(15)

C12C11C8	120.24(14)

C16C11C8	122.07(14)

C13C12C11	120.98(15)

C13C12H12	119.5

C11C12H12	119.5

C12C13C14	121.34(15)

C12C13H13	119.3

C14C13H13	119.3

C15C14C13	117.88(15)

C15C14C17	121.80(16)

C13C14C17	120.32(16)

C16C15C14	120.96(15)

C16C15H15	119.5

C14C15H15	119.5

C15C16C11	121.23(16)

C15C16H16	119.4

C11C16H16	119.4

C14C17H17A	109.5

C14C17H17B	109.5

H17AC17H17B	109.5

C14C17H17C	109.5

H17AC17H17C	109.5

H17BC17H17C	109.5

C1O1H1	109.5

Symmetry transformations used to generate equivalent atoms:

Table 4. Anisotropic displacement parameters [Å2 103]. The anisotropic displacement

factor exponent takes the form: 2 2[h2a*2U11 + ... + 2 h k a* b* U12 ].

Atom	U11	U22	U33	U23	U13	U12

C1	29(1) 	25(1)	22(1) 	4(1)	4(1) 	0(1)

C2	34(1) 	33(1)	25(1) 	4(1)	9(1) 	7(1)

C3	46(1) 	30(1)	24(1) 	2(1)	8(1) 	7(1)

C4	46(1) 	27(1)	23(1) 	2(1)	2(1) 	2(1)

C5	33(1) 	30(1)	27(1) 	4(1)	0(1) 	9(1)

C6	23(1) 	40(1)	31(1) 	6(1)	3(1) 	5(1)

C7	26(1) 	30(1)	30(1) 	4(1)	7(1) 	3(1)

C8	26(1) 	22(1)	22(1) 	3(1)	4(1) 	0(1)

C9	27(1) 	19(1)	19(1) 	3(1)	3(1) 	0(1)

C10	34(1) 	23(1)	19(1) 	4(1)	1(1) 	3(1)

C11	25(1) 	23(1)	24(1) 	1(1)	6(1) 	1(1)

C12	36(1) 	26(1)	23(1) 	2(1)	3(1) 	1(1)

C13	37(1) 	21(1)	29(1) 	0(1)	7(1) 	0(1)

C14	25(1) 	27(1)	29(1) 	6(1)	9(1) 	1(1)

C15	28(1) 	34(1)	25(1) 	1(1)	2(1) 	1(1)

C16	31(1) 	25(1)	27(1) 	4(1)	4(1) 	3(1)

C17	36(1) 	33(1)	40(1) 	10(1)	7(1) 	8(1)

O1	27(1) 	36(1)	43(1) 	9(1)	10(1) 	4(1)

Thermal ellipsoids drawn at the 50% probability level.