|  | # Departmental Single Crystal X-Ray Diffraction Service School of Chemistry - University of Southampton Contact: Dr Mark E Light, light@soton.ac.uk, ex 29429 |
| --- | --- |

Table 1. Crystal data and structure refinement details.

Identification code 	06skc0034p21c

Empirical formula 	C7H10N2O4

Formula weight 	186.17

Temperature 	120(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P21/c

Unit cell dimensions	a = 6.4624(4) Å	 = 90°

b = 18.8186(10) Å	 = 98.878(4)°

c = 7.1493(5) Å	  = 90°

Volume	859.03(9) Å3

Z	4

Density (calculated)	1.439 Mg / m3

Absorption coefficient	0.119 mm1

F(000)	392

Crystal	lath; colourless

Crystal size	0.06  0.05  0.01 mm3

 range for data collection	3.19  27.48°

Index ranges	8  h  8, 24  k  24, 9  l  9

Reflections collected	11322

Independent reflections	1972 [Rint = 0.1031]

Completeness to  = 27.48°	99.7 %

Max. and min. transmission	0.9988 and 0.9929

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	1972 / 0 / 121

Goodness-of-fit on F2	1.044

Final R indices [F2 > 2(F2)]	R1 = 0.0609, wR2 = 0.1067

R indices (all data)	R1 = 0.1145, wR2 = 0.1230

Largest diff. peak and hole	0.225 and 0.261 e Å3

Diffractometer: Nonius KappaCCD area detector ( scans and  scans to fill asymmetric unit ). Cell determination: DirAx (Duisenberg, A.J.M.(1992). J. Appl. Cryst. 25, 92-96.) Data collection: Collect (Collect: Data collection software, R. Hooft, Nonius B.V., 1998). Data reduction and cell refinement: Denzo (Z. Otwinowski & W. Minor, Methods in Enzymology (1997) Vol. 276: Macromolecular Crystallography, part A, pp. 307326; C. W. Carter, Jr. & R. M. Sweet, Eds., Academic Press). Absorption correction: Sheldrick, G. M. SADABS - Bruker Nonius area detector scaling and absorption correction - V2.10 Structure solution: SHELXS97 (G. M. Sheldrick, Acta Cryst. (1990) A46 467473). Structure refinement: SHELXL97 (G. M. Sheldrick (1997), University of Göttingen, Germany). Graphics: Cameron - A Molecular Graphics Package. (D. M. Watkin, L. Pearce and C. K. Prout, Chemical Crystallography Laboratory, University of Oxford, 1993).

Special details:

Table 2. Atomic coordinates [ 104], equivalent isotropic displacement parameters [Å2  103] and site occupancy factors. Ueq is defined as one third of the trace of the orthogonalized Uij tensor.

Atom 	x	y	z	Ueq	S.o.f.

C1	4337(4)	5652(1)	6478(3)	20(1)	1

C2	6509(4)	5977(1)	7321(3)	20(1)	1

C3	1090(4)	5878(1)	4594(4)	27(1)	1

O1A	3773(3)	5064(1)	6826(2)	27(1)	1

O1B	3184(2)	6116(1)	5367(2)	23(1)	1

O2A	7714(2)	5545(1)	8312(2)	25(1)	1

O2B	6838(3)	6602(1)	6971(2)	28(1)	1

C22	13045(3)	6030(1)	10561(3)	19(1)	1

C24	11270(4)	6973(1)	9453(3)	24(1)	1

C25	13182(4)	7196(1)	10289(4)	25(1)	1

C26	16411(4)	6589(1)	12006(3)	25(1)	1

N21	14261(3)	6596(1)	10975(3)	19(1)	1

N23	11217(3)	6249(1)	9634(3)	22(1)	1

Table 3. Bond lengths [Å] and angles [°].

C1O1A	1.202(3)

C1O1B	1.329(3)

C1C2	1.564(3)

C2O2B	1.228(3)

C2O2A	1.264(3)

C3O1B	1.452(3)

C3H3A	0.9800

C3H3B	0.9800

C3H3C	0.9800

C22N23	1.328(3)

C22N21	1.329(3)

C22H22	0.9500

C24C25	1.353(3)

C24N23	1.369(3)

C24H24	0.9500

C25N21	1.376(3)

C25H25	0.9500

C26N21	1.469(3)

C26H26A	0.9800

C26H26B	0.9800

C26H26C	0.9800

N23H23	0.8800

O1AC1O1B	124.6(2)

O1AC1C2	124.2(2)

O1BC1C2	111.1(2)

O2BC2O2A	128.3(2)

O2BC2C1	118.0(2)

O2AC2C1	113.8(2)

O1BC3H3A	109.5

O1BC3H3B	109.5

H3AC3H3B	109.5

O1BC3H3C	109.5

H3AC3H3C	109.5

H3BC3H3C	109.5

C1O1BC3	115.36(18)

N23C22N21	108.2(2)

N23C22H22	125.9

N21C22H22	125.9

C25C24N23	107.5(2)

C25C24H24	126.2

N23C24H24	126.2

C24C25N21	106.3(2)

C24C25H25	126.8

N21C25H25	126.8

N21C26H26A	109.5

N21C26H26B	109.5

H26AC26H26B	109.5

N21C26H26C	109.5

H26AC26H26C	109.5

H26BC26H26C	109.5

C22N21C25	109.13(19)

C22N21C26	125.87(19)

C25N21C26	125.0(2)

C22N23C24	108.8(2)

C22N23H23	125.6

C24N23H23	125.6

Symmetry transformations used to generate equivalent atoms:

Table 4. Anisotropic displacement parameters [Å2 103]. The anisotropic displacement

factor exponent takes the form: 2 2[h2a*2U11 + ... + 2 h k a* b* U12 ].

Atom	U11	U22	U33	U23	U13	U12

C1	24(1) 	16(1)	18(1) 	2(1)	3(1) 	1(1)

C2	20(1) 	19(1)	22(1) 	0(1)	2(1) 	1(1)

C3	18(1) 	27(1)	34(2) 	1(1)	6(1) 	3(1)

O1A	26(1) 	22(1)	31(1) 	4(1)	1(1) 	4(1)

O1B	18(1) 	18(1)	30(1) 	2(1)	3(1) 	1(1)

O2A	22(1) 	20(1)	30(1) 	3(1)	5(1) 	0(1)

O2B	26(1) 	19(1)	36(1) 	3(1)	5(1) 	4(1)

C22	21(1) 	15(1)	21(1) 	0(1)	2(1) 	1(1)

C24	25(1) 	18(1)	27(1) 	5(1)	0(1) 	2(1)

C25	26(1) 	16(1)	31(2) 	1(1)	0(1) 	0(1)

C26	18(1) 	28(1)	27(1) 	2(1)	3(1) 	2(1)

N21	18(1) 	18(1)	19(1) 	0(1)	1(1) 	0(1)

N23	20(1) 	21(1)	22(1) 	1(1)	2(1) 	3(1)

Table 5. Hydrogen bonds [Å and °].

DH···A	d(DH)	d(H···A)	d(D···A)	(DHA)

N23H23...O2A	0.88	1.80	2.667(3)	170.5

Symmetry transformations used to generate equivalent atoms: