|  | University of Southampton · Department of Chemistry # EPSRC National Crystallography Service |  |
| --- | --- | --- |

Table 1. Crystal data and structure refinement.

Identification code 	00mau005

Empirical formula 	C6H9N3O3

Formula weight 	171.16

Temperature 	150(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P21/c

Unit cell dimensions	a = 6.9056(14) Å	 = 90°

b = 8.6519(17) Å	 = 94.41(3)°

c = 12.818(3) Å	  = 90°

Volume	763.6(3) Å3

Z	4

Density (calculated)	1.489 Mg / m3

Absorption coefficient	0.121 mm1

F(000)	360

Crystal	needle; orange

Crystal size	0.40  0.12  0.08 mm3

 range for data collection	2.96  27.48°

Index ranges	8  h  8, 11  k  11, 16  l  16

Reflections collected	6083

Independent reflections	1725 [Rint = 0.0378]

Completeness to  = 27.48°	98.8 %

Absorption correction	Semiempirical from equivalents

Max. and min. transmission	0.9904 and 0.9532

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	1725 / 0 / 112

Goodness-of-fit on F2	1.021

Final R indices [F2 > 2(F2)]	R1 = 0.0397, wR2 = 0.1010

R indices (all data)	R1 = 0.0547, wR2 = 0.1099

Extinction coefficient	0.042(10)

Largest diff. peak and hole	0.202 and 0.242 e Å3

Diffractometer: Enraf Nonius KappaCCD area detector ( scans and  scans to fill Ewald sphere). Data collection and cell

refinement: Denzo (Z. Otwinowski & W. Minor, Methods in Enzymology (1997) Vol. 276: Macromolecular Crystallography,

part A, pp. 307326; C. W. Carter, Jr. & R. M. Sweet, Eds., Academic Press). Absorption correction: SORTAV (R. H. Blessing,

Acta Cryst. A51 (1995) 3337; R. H. Blessing, J. Appl. Cryst. 30 (1997) 421426). Program used to solve structure:

SHELXS97 (G. M. Sheldrick, Acta Cryst. (1990) A46 467473). Program used to refine structure: SHELXL97 (G. M. Sheldrick

(1997), University of Göttingen, Germany).

Further information:  http://www.soton.ac.uk/~xservice/strat.htm

Special details:

Table 2. Atomic coordinates [ 104], equivalent isotropic displacement parameters [Å2  103] and site occupancy factors. Ueq is defined as one third of the trace of the orthogonalized Uij tensor.

Atom 	x	y	z	Ueq	S.o.f.

C1	2166(2)	2124(2)	5360(1)	23(1)	1

C2	2125(2)	3687(2)	5820(1)	34(1)	1

C3	3710(2)	832(2)	6964(1)	26(1)	1

C4	2276(2)	253(2)	7713(1)	28(1)	1

C5	2685(2)	331(2)	5153(1)	21(1)	1

C6	1859(2)	257(2)	4243(1)	24(1)	1

N1	1534(2)	1790(1)	4373(1)	26(1)	1

N2	2882(1)	859(1)	5874(1)	21(1)	1

N3	3227(2)	1891(1)	5351(1)	26(1)	1

O1	606(1)	1200(1)	7701(1)	31(1)	1

O2	3036(2)	2783(1)	4603(1)	39(1)	1

O3	3849(2)	2284(1)	6232(1)	39(1)	1

Table 3. Bond lengths [Å] and angles [°].

C1N1	1.3378(17)

C1N2	1.3514(16)

C1C2	1.477(2)

C3N2	1.4696(16)

C3C4	1.5159(19)

C4O1	1.4133(17)

C5C6	1.3577(18)

C5N2	1.3826(16)

C5N3	1.4181(18)

C6N1	1.3580(19)

N3O3	1.2248(15)

N3O2	1.2300(15)

N1C1N2	111.49(11)

N1C1C2	124.10(12)

N2C1C2	124.40(12)

N2C3C4	112.46(10)

O1C4C3	111.94(11)

C6C5N2	107.93(12)

C6C5N3	126.72(12)

N2C5N3	125.34(11)

C5C6N1	108.86(11)

C1N1C6	106.37(11)

C1N2C5	105.34(10)

C1N2C3	125.24(11)

C5N2C3	129.41(10)

O3N3O2	123.60(12)

O3N3C5	119.87(11)

O2N3C5	116.53(11)

Symmetry transformations used to generate equivalent atoms:

Table 4. Anisotropic displacement parameters [Å2 103]. The anisotropic displacement

factor exponent takes the form: 2 2[h2a*2U11 + ... + 2 h k a* b* U12 ].

Atom	U11	U22	U33	U23	U13	U12

C1	22(1) 	24(1)	24(1) 	3(1)	3(1) 	2(1)

C2	42(1) 	24(1)	37(1) 	1(1)	2(1) 	0(1)

C3	28(1) 	32(1)	18(1) 	2(1)	5(1) 	1(1)

C4	37(1) 	28(1)	20(1) 	1(1)	0(1) 	2(1)

C5	20(1) 	22(1)	21(1) 	1(1)	2(1) 	2(1)

C6	23(1) 	29(1)	19(1) 	1(1)	2(1) 	5(1)

N1	26(1) 	28(1)	22(1) 	4(1)	1(1) 	1(1)

N2	22(1) 	22(1)	19(1) 	1(1)	0(1) 	1(1)

N3	26(1) 	26(1)	26(1) 	2(1)	3(1) 	0(1)

O1	32(1) 	36(1)	24(1) 	8(1)	0(1) 	1(1)

O2	56(1) 	27(1)	34(1) 	11(1)	3(1) 	0(1)

O3	52(1) 	33(1)	31(1) 	4(1)	7(1) 	12(1)

Table 5. Hydrogen coordinates [ 104] and isotropic displacement parameters [Å2  103].

Atom 	x	y	z	Ueq	S.o.f.

H2A	1516	4408	5304	51	1

H2B	3455	4026	6022	51	1

H2C	1377	3666	6439	51	1

H3A	4132	1889	7173	31	1

H3B	4871	157	7017	31	1

H4A	1880	814	7518	34	1

H4B	2916	226	8431	34	1

H6	1557	312	3618	28	1

H1	748	1853	8186	46	1